HAND TAP CONTROLLED AC DEVICE

The project implements a real-time hand-gesture-based human–computer interaction system that controls both the computer cursor and external AC devices via an ESP32 microcontroller.​

abstract

The system, “Hand Gesture–Driven Human–Computer Interaction for AC Device Control,” uses a webcam, MediaPipe Hand landmarks, and a Kalman filter to track the index fingertip and move the on-screen cursor smoothly, while thumb–index contact is used as a click/hold gesture. An ESP32 sends serial commands such as “scroll_up” and “scroll_down” that the PC converts into scrolling actions, enabling hybrid control via both camera-based hand gestures and external sensors.​

Detailed system description

- Sensing and tracking

  - Captures video from a webcam at 60 FPS using OpenCV’s VideoCapture.​

  - Uses MediaPipe Hands with one-hand tracking, minimum detection and tracking confidence of 0.7 to obtain 21 hand landmarks per frame.​

  - Extracts thumb tip (landmark 4) and index fingertip (landmark 8) positions in pixel coordinates relative to the frame size.​

- Gesture detection logic

  - Implements is_touch_hold, which computes the absolute vertical distance between thumb tip and index tip using normalized y-coordinates, and treats them as “touching/holding” if this distance is below a threshold (0.04).​

  - When “touch and hold” is detected, the system issues pyautogui.mouseDown(), and when the gesture is released it calls pyautogui.mouseUp(), effectively emulating mouse click-and-hold using thumb–index contact.​

- Kalman-filter-based smoothing and cursor control

  - Defines a 4D Kalman filter state [x,y,vx,vy][x,y,vx,vy] with a constant-velocity transition model to smooth noisy fingertip positions.​

  - Each frame, the current index fingertip position is passed as a measurement; the Kalman filter predicts and corrects the state, returning a smoothed (x,y)(x,y) used as the cursor position.​

  - Cursor movement is executed by pyautogui.moveTo, with a dynamically adjusted duration based on hand movement speed: faster hand motion increases a speed factor that shortens the movement duration for more responsive control.​

- Serial communication with ESP32 for device-level interaction

  - Opens a serial connection on COM3 at 9600 baud to communicate with an ESP32; if the port fails to open, the program prints an error and exits to avoid undefined behavior.​

  - Continuously monitors ser.in_waiting; when data is available, it reads and decodes a line and checks for textual commands such as scroll_up and scroll_down.​

  - On recognizing these commands, it calls pyautogui.scroll(1) or pyautogui.scroll(-1), mapping the ESP32’s external sensor events to scrolling actions in the GUI environment.​

Implementation flow

- Initialize GPU for TensorFlow if available, enabling memory growth to prevent allocation errors when other GPU-based modules are used in the system.​

- Set up camera, MediaPipe Hands, Kalman filter, and serial link, then enter a loop that:

  - Captures and flips frames, processes hand landmarks, updates Kalman-filtered fingertip position, and moves the cursor.

  - Checks the thumb–index distance to trigger mouse down/up events.

  - Reads ESP32 serial commands to trigger scroll events.

  - Renders hand landmarks on the video and displays the “Hand Gesture Control” window until the Escape key is pressed, then releases camera and serial resources and closes all windows.​

Role in the overall project report

- This implementation section concretely realizes the project’s goal of replacing traditional input with gesture-based interaction by:

  - Mapping physiological signals (hand movements and thumb–index contact) to cursor motion and click behavior in real time.​

  - Integrating external microcontroller input (ESP32 serial commands) with GUI control, providing a bridge to AC device control and additional gestures sensed in hardware.​

- It directly supports the project’s focus on usability, accuracy, and responsiveness through smoothing (Kalman filter), adaptive speed control, and stable click detection, forming the technical core of the proposed gesture-based interaction paradigm.​

CODE

import cv2

import mediapipe as mp

import pyautogui

import serial

import numpy as np

from collections import deque

import tensorflow as tf

# Check if GPU is available and set memory growth

gpus = tf.config.experimental.list_physical_devices('GPU')

if gpus:

try:

for gpu in gpus:

tf.config.experimental.set_memory_growth(gpu, True)

logical_gpus = tf.config.experimental.list_logical_devices('GPU')

print(len(gpus), "Physical GPUs,", len(logical_gpus), "Logical GPUs")

except RuntimeError as e:

print(e)

# Function to detect thumb and index finger touching and holding

def is_touch_hold(hand_keypoints, touch_thresh=0.04):

thumb_top_y = hand_keypoints.landmark[4].y

index_top_y = hand_keypoints.landmark[8].y

thumb_index_dist = abs(thumb_top_y - index_top_y)

return thumb_index_dist < touch_thresh

# Kalman filter for smoothing hand position

class KalmanFilter:

def __init__(self, process_noise=0.1, measurement_noise=0.1):

self.state = np.array([0, 0, 0, 0])  # [x, y, vx, vy]

self.transition_matrix = np.array([[1, 0, 1, 0], [0, 1, 0, 1], [0, 0, 1, 0], [0, 0, 0, 1]])

self.observation_matrix = np.array([[1, 0, 0, 0], [0, 1, 0, 0]])

self.process_noise_covariance = np.eye(4) * process_noise

self.measurement_noise_covariance = np.eye(2) * measurement_noise

self.error_covariance = np.eye(4)

def update(self, measurement):

# Predict

self.state = np.dot(self.transition_matrix, self.state)

self.error_covariance = np.dot(np.dot(self.transition_matrix, self.error_covariance), self.transition_matrix.T) + self.process_noise_covariance

# Update

kalman_gain = np.dot(np.dot(self.error_covariance, self.observation_matrix.T),

np.linalg.inv(np.dot(np.dot(self.observation_matrix, self.error_covariance), self.observation_matrix.T) + self.measurement_noise_covariance))

self.state = self.state + np.dot(kalman_gain, measurement - np.dot(self.observation_matrix, self.state))

self.error_covariance = (np.eye(4) - np.dot(kalman_gain, self.observation_matrix)) @ self.error_covariance

return self.state[:2]

# Initialize VideoCapture and MediaPipe Hands

cap = cv2.VideoCapture(0)

cap.set(cv2.CAP_PROP_FPS, 60)  # Set the frame rate to 60 FPS

mp_hands = mp.solutions.hands

hands = mp_hands.Hands(max_num_hands=1, min_detection_confidence=0.7, min_tracking_confidence=0.7)

mp_drawing = mp.solutions.drawing_utils

# Initialize serial communication with ESP32

try:

ser = serial.Serial('COM3', 9600)  # Replace 'COM3' with the correct port

except serial.SerialException as e:

print("Error:", e)

exit()

# Previous hand position for smoothing

prev_hand_pos = None

hand_positions = deque(maxlen=10)  # Store the last 10 hand positions for smoothing

kalman_filter = KalmanFilter(process_noise=0.1, measurement_noise=0.1)

# Main loop

while True:

ret, frame = cap.read()

if not ret:

print("Error: Unable to capture frame")

break

frame = cv2.flip(frame, 1)

rgb_frame = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)

results = hands.process(rgb_frame)

if results.multi_hand_landmarks:

hand_landmarks = results.multi_hand_landmarks[0]

# Calculate the current hand position

thumb_tip_x = int(hand_landmarks.landmark[4].x * frame.shape[1])

thumb_tip_y = int(hand_landmarks.landmark[4].y * frame.shape[0])

index_tip_x = int(hand_landmarks.landmark[8].x * frame.shape[1])

index_tip_y = int(hand_landmarks.landmark[8].y * frame.shape[0])

current_hand_pos = (index_tip_x, index_tip_y)

# Store the current hand position in the deque

hand_positions.append(current_hand_pos)

# Smooth cursor movement using Kalman filtering

smoothed_hand_pos = kalman_filter.update(np.array([current_hand_pos[0], current_hand_pos[1]]))

smoothed_hand_pos = (int(smoothed_hand_pos[0]), int(smoothed_hand_pos[1]))

# Adjust cursor speed based on hand movement

hand_movement_speed = np.linalg.norm(np.array(current_hand_pos) - np.array(prev_hand_pos)) if prev_hand_pos else 0

cursor_speed_factor = 1 + 0.5 * (hand_movement_speed / 100)  # Adjust the speed factor as needed

pyautogui.moveTo(smoothed_hand_pos[0], smoothed_hand_pos[1], duration=0.01 / cursor_speed_factor)  # Adjust the duration for smoother movement

prev_hand_pos = current_hand_pos

if is_touch_hold(hand_landmarks):

pyautogui.mouseDown()

else:

pyautogui.mouseUp()

mp_drawing.draw_landmarks(frame, hand_landmarks, mp_hands.HAND_CONNECTIONS)

if ser.in_waiting > 0:

serial_data = ser.readline().decode().strip()

print("Received:", serial_data)

if serial_data == "scroll_up":

pyautogui.scroll(1)  # Scroll up

elif serial_data == "scroll_down":

pyautogui.scroll(-1)  # Scroll down

cv2.imshow("Hand Gesture Control", frame)

if cv2.waitKey(1) == 27:

break

# Release resources

cap.release()

ser.close()

cv2.destroyAllWindows()